SIM7600G-H

Developer Reference

Calls · SMS · AT Commands · C Integration · Black Hand OS

Black Hand OS  ·  Phase 2

# 1. Hardware Overview

The SIM7600G-H is a multi-band LTE Cat-4 module made by SIMCom. It supports LTE, WCDMA, and GSM fallback. On Black Hand OS it connects to the Raspberry Pi 4B over USB (a CDC-ACM serial port), giving you AT command control over calls, SMS, data, GPS, and audio.

## 1.1 Key Specifications

| Parameter | Value |
| --- | --- |
| Form factor | LCC + LGA, 64.0 × 64.0 × 4.2 mm |
| Supply voltage | 3.3 – 4.2 V (3.8 V nominal) |
| LTE Cat | Cat-4 (150 Mbps DL / 50 Mbps UL) |
| LTE bands (global) | B1/2/3/4/5/7/8/12/13/18/19/20/25/26/28/38/39/40/41 |
| WCDMA bands | B1/2/4/5/6/8/19 |
| GSM/GPRS | 850 / 900 / 1800 / 1900 MHz |
| USB interface | USB 2.0 High Speed — CDC-ACM + ECM |
| UART interface | 3.3 V logic, up to 3 Mbps |
| Audio codec | Built-in — PCM, I2S, analog mic/speaker |
| GNSS | GPS / GLONASS / BeiDou / Galileo |
| Operating temp | -40 °C to +85 °C |
| SIM | 1.8 V / 3.0 V, Nano SIM |

## 1.2 USB Interface on Raspberry Pi 4B

When plugged into the Pi over USB, the SIM7600G-H enumerates several CDC-ACM ports. Their exact /dev paths depend on insertion order, but on a clean boot they are typically:

| Port | Purpose |
| --- | --- |
| /dev/ttyUSB0 | AT command port (primary — use this for calls and SMS) |
| /dev/ttyUSB1 | AT command port (secondary / GPS NMEA, board-dependent) |
| /dev/ttyUSB2 | Diagnostic / audio PCM |
| /dev/ttyUSB3 | Modem data (PPP / MBIM) |

⚠ WARNING: Port numbers can shift

If you have other USB serial devices (e.g. Arduino, FTDI) plugged in, ttyUSB0 may belong to them. Always confirm the correct port by sending AT and checking for OK. In Black Hand OS, blackhand-modem iterates /dev/ttyUSB* until it gets a valid AT response.

## 1.3 UART Interface (Alternative)

If you need a dedicated serial line without the USB enumeration overhead, the module's UART_TXD / UART_RXD pins (3.3 V logic) connect to the Pi's GPIO UART. This is the same UART ttyAMA0 that the Pi 4 uses for Bluetooth, so you would need to disable Bluetooth or add an external UART HAT.

For Black Hand OS we use USB exclusively, keeping Bluetooth on ttyAMA0.

## 1.4 SIM Card

The SIM7600G-H requires a Nano SIM (4FF). The SIM must be inserted before powering the module. If you hot-swap the SIM, send AT+CFUN=1,1 to soft-reset the modem and reload the SIM.

# 2. AT Command Fundamentals

All interaction with the modem goes through AT commands over the serial port. Every command starts with AT (attention) and ends with a carriage-return (\r, 0x0D). Responses arrive as text terminated by \r\n.

## 2.1 Command Syntax

| Type | Syntax | Example |
| --- | --- | --- |
| Test | AT+CMD=? | AT+CMGF=? |
| Read | AT+CMD? | AT+CMGF? |
| Set | AT+CMD=value | AT+CMGF=1 |
| Execute | AT+CMD | AT+CSQ |
| Basic | ATX | ATZ, ATE0 |

## 2.2 Response Codes

| Response | Meaning |
| --- | --- |
| OK | Command accepted and executed |
| ERROR | Command rejected (malformed or not allowed in current state) |
| +CME ERROR: <n> | Mobile Equipment error (network / SIM errors) |
| +CMS ERROR: <n> | SMS-specific error |
| CONNECT | Data call connected |
| NO CARRIER | Call ended / line not available |
| BUSY | Remote line busy |
| NO ANSWER | Call timed out without answer |
| RING | Incoming call signalled |

## 2.3 Echo and Verbosity

Initial modem setup — send these first on every connection

ATE0 ; disable echo (don't echo back what we send) ATQ0 ; enable result codes (OK / ERROR returned) ATV1 ; verbose result codes (text, not numeric) AT+CMEE=2 ; extended error reporting (text descriptions)

## 2.4 Opening the Port from Linux / C

C — open /dev/ttyUSB0 at 115200 baud, raw mode

#include <termios.h> #include <fcntl.h> #include <unistd.h> int open_modem(const char *dev) { int fd = open(dev, O_RDWR ¦ O_NOCTTY ¦ O_NONBLOCK); if (fd < 0) return -1; struct termios t = {0}; cfsetispeed(&t, B115200); cfsetospeed(&t, B115200); t.c_cflag = CS8 ¦ CLOCAL ¦ CREAD; // 8N1 t.c_iflag = 0; // no input processing t.c_oflag = 0; // no output processing t.c_lflag = 0; // raw mode t.c_cc[VMIN] = 0; t.c_cc[VTIME] = 10; // 1 second timeout tcsetattr(fd, TCSANOW, &t); tcflush(fd, TCIOFLUSH); return fd; }

C — send AT command and read response

// Returns pointer to static buffer containing response, or NULL on timeout. // Waits up to timeout_ms for final result code (OK / ERROR / +CME). const char *at_cmd(int fd, const char *cmd, int timeout_ms) { static char buf[4096]; char tmp[256]; snprintf(tmp, sizeof(tmp), "%s\r", cmd); write(fd, tmp, strlen(tmp)); buf[0] = 0; int total = 0; long deadline = now_ms() + timeout_ms; while (now_ms() < deadline) { int n = read(fd, buf + total, sizeof(buf) - total - 1); if (n > 0) { total += n; buf[total] = 0; } if (strstr(buf, "\r\nOK\r\n") ¦¦ strstr(buf, "\r\nERROR\r\n") ¦¦ strstr(buf, "+CME ERROR") ¦¦ strstr(buf, "+CMS ERROR")) break; usleep(10000); } return buf; }

## 2.5 Essential Status Commands

| Command | Response Example | What It Checks |
| --- | --- | --- |
| AT | OK | Modem alive |
| AT+CSQ | +CSQ: 18,0 | Signal quality (0-31, 99=unknown) |
| AT+CREG? | +CREG: 0,1 | Network registration (1=home, 5=roaming) |
| AT+CGREG? | +CGREG: 0,1 | GPRS registration |
| AT+CEREG? | +CEREG: 0,1 | LTE registration |
| AT+COPS? | +COPS: 0,0,"T-Mobile" | Current operator |
| AT+CIMI | 310260123456789 | IMSI (SIM identity) |
| AT+CGSN | 866123456789012 | IMEI (modem identity) |
| AT+CNUM | +CNUM: ,"+447700900000",145 | Own phone number |
| AT+CFUN? | +CFUN: 1 | Functionality level (1=full) |
| AT+CPAS | +CPAS: 0 | Activity status (0=ready, 3=ringing, 4=call) |

# 3. Voice Calls

Voice calls on the SIM7600G-H use circuit-switched telephony. The audio path goes through the modem's onboard codec — you do NOT stream PCM over UART yourself. Instead you route audio through the modem's speaker/mic pins, or configure a PCM/I2S digital audio interface.

## 3.1 Making an Outgoing Call

Dial a number

ATD+447700900001; ; note the semicolon — it means voice call (not data) ; Response: OK (dialling) → then either: ; CONNECT (answered) or NO CARRIER / BUSY / NO ANSWER

✖ CRITICAL: The semicolon is mandatory for voice calls

ATD<number>; — the trailing semicolon tells the modem to make a VOICE call and stay in command mode. Without it the modem attempts a DATA call and enters online mode, which will break AT command processing.

C — make an outgoing call

int dial(int fd, const char *number) { char cmd[64]; snprintf(cmd, sizeof(cmd), "ATD%s;", number); const char *r = at_cmd(fd, cmd, 2000); if (strstr(r, "OK")) return 0; // dialling in progress if (strstr(r, "ERROR")) return -1; // number invalid / modem busy return -1; } // After dial() returns 0, monitor for URC (unsolicited result codes): // 'NO CARRIER' = call dropped / rejected // 'BUSY' = remote busy // 'NO ANSWER' = ring timeout // To check call status actively: AT+CLCC

## 3.2 Checking Active Call Status — AT+CLCC

AT+CLCC (List Current Calls) returns all calls and their states. Poll it after dialling to track the call lifecycle.

AT+CLCC response format

+CLCC: <idx>,<dir>,<stat>,<mode>,<mpty>[,<number>,<type>[,<alpha>]] idx = call index (1-7) dir = 0 mobile-originated, 1 mobile-terminated stat = 0 active, 1 held, 2 dialling, 3 alerting (ringing out), 4 incoming, 5 waiting mode = 0 voice, 1 data, 2 fax mpty = 0 not multiparty, 1 multiparty Example: +CLCC: 1,0,3,0,0,"+447700900001",145 (call 1, outgoing, alerting=ringing, voice, not multiparty)

## 3.3 Answering an Incoming Call

When an incoming call arrives, the modem sends a RING URC (and possibly +CLIP for caller ID). You answer with ATA.

Incoming call sequence

// Modem sends spontaneously: RING +CLIP: "+447700900002",145,,,,0 ; caller ID (if AT+CLIP=1 enabled) RING RING // Your code sends: ATA // Modem responds: OK

C — enable caller ID and answer call

// One-time setup: enable caller ID presentation at_cmd(fd, "AT+CLIP=1", 2000); // In your URC reader loop: if (strstr(urc_line, "RING")) { // extract caller from +CLIP line if present at_cmd(fd, "ATA", 2000); // answer }

## 3.4 Hanging Up

Terminate any call

ATH ; hang up active call AT+CHUP ; hang up (same as ATH, more explicit) // Reject an incoming call without answering: ATH ; works for both active and incoming

## 3.5 Call Waiting and Hold — AT+CHLD

When you have an active call and a second incoming call arrives, use AT+CHLD to manage them.

| Command | Action |
| --- | --- |
| AT+CHLD=0 | Reject waiting call / release held calls |
| AT+CHLD=1 | Release active call, accept waiting or held |
| AT+CHLD=2 | Hold active call, accept waiting (call swap) |
| AT+CHLD=3 | Merge active + held into conference |

## 3.6 DTMF Tones — AT+VTS

To send DTMF tones during an active voice call (e.g. to navigate IVR menus):

Send DTMF tones

AT+VTS=1 ; send digit 1 AT+VTS=* ; send * AT+VTS={1,1,200} ; send '1' for 200 ms // Multi-tone sequence: AT+VTS=1;+VTS=2;+VTS=3 ; send 1, 2, 3 in sequence

## 3.7 Audio Routing

The SIM7600G-H has three audio paths. For Black Hand OS with a USB audio dongle the recommended approach is to use the PCM digital audio interface, which routes audio through the modem codec and your external DAC/ADC simultaneously.

| Audio Mode | Command | Notes |
| --- | --- | --- |
| Analog mic/spk | AT+CSDVC=1 | Uses pins MICP/MICN, SPK+/SPK- |
| Handsfree spk | AT+CSDVC=3 | Loudspeaker output |
| PCM interface | AT+CPCMREG=1 | Digital I2S/PCM — best for Pi integration |
| Volume (0-5) | AT+CLVL=4 | Speaker gain |
| Mic mute | AT+CMUT=1 | Mute microphone |

✓ TIP: Audio for Black Hand OS

For a USB audio dongle setup: route modem audio via PCM interface to a DAC/ADC chip wired to the Pi's GPIO I2S pins, OR keep analog and wire the modem's SPK+/SPK- directly to an amplifier. The simplest approach is analog direct — no kernel driver needed.

## 3.8 Complete Outgoing Call Flow — C Implementation

call_manager.c — simplified complete call flow

typedef enum { CALL_IDLE, CALL_DIALLING, CALL_ACTIVE, CALL_HELD } CallState; static CallState s_state = CALL_IDLE; // Poll this in your main loop to process URCs void modem_poll(int fd) { char buf[512]; int n = read(fd, buf, sizeof(buf)-1); if (n <= 0) return; buf[n] = 0; // Incoming call if (strstr(buf, "RING")) { if (s_state == CALL_IDLE) { s_state = CALL_DIALLING; // repurpose for 'ringing in' ui_show_incoming_call(); } } // Call connected (outgoing answered / incoming accepted) if (strstr(buf, "+CLCC") && strstr(buf, ",0,")) { // stat=0 active s_state = CALL_ACTIVE; ui_show_active_call(); } // Call ended if (strstr(buf, "NO CARRIER") ¦¦ strstr(buf, "BUSY") ¦¦ strstr(buf, "NO ANSWER")) { s_state = CALL_IDLE; ui_show_call_ended(); } } // Make a call int make_call(int fd, const char *number) { if (s_state != CALL_IDLE) return -1; // already in call char cmd[64]; snprintf(cmd, sizeof(cmd), "ATD%s;", number); const char *r = at_cmd(fd, cmd, 3000); if (!strstr(r, "OK")) return -1; s_state = CALL_DIALLING; return 0; } // Hang up void hang_up(int fd) { at_cmd(fd, "ATH", 2000); s_state = CALL_IDLE; } // Answer incoming call void answer_call(int fd) { at_cmd(fd, "ATA", 2000); s_state = CALL_ACTIVE; }

# 4. SMS

The SIM7600G-H supports two SMS modes: Text mode (human-readable AT commands) and PDU mode (binary protocol giving full control over encoding, UDH, multipart). For Black Hand OS, Text mode is sufficient for standard messaging.

## 4.1 Text Mode vs PDU Mode

| Feature | Text Mode | PDU Mode |
| --- | --- | --- |
| Setup | Simple | Complex — encode bytes manually |
| Encoding | GSM7 or UCS2 | Full control (7-bit, 8-bit, UCS2) |
| Long SMS | Auto-split by modem | You construct UDH headers |
| Emoji / Unicode | AT+CSCS="UCS2" | Full Unicode control |
| Read received SMS | Decoded text | Raw binary — you decode |
| Best for | Most use cases | Custom encoding / binary SMS |

## 4.2 Setup for Text Mode

One-time SMS configuration

AT+CMGF=1 ; select Text mode (0 = PDU mode) AT+CSCS="GSM" ; character set: GSM7 (standard ASCII-like) ; use "UCS2" for emoji / non-Latin scripts AT+CNMI=2,2,0,0,0 ; new message indication: route directly to serial port ; (2,2) = route +CMT URCs directly, not to storage AT+CPMS="SM","SM","SM" ; use SIM storage (or "ME" for modem memory)

## 4.3 Sending an SMS

Send SMS — AT command sequence

AT+CMGS="+447700900001" ; start SMS to this number > Hello from Black Hand OS ; modem prompts with '>'; type message then: <CTRL-Z> ; 0x1A byte — commits and sends ; Response: +CMGS: <ref> then OK

C — send SMS function

#define CTRL_Z "\x1A" int sms_send(int fd, const char *number, const char *text) { char cmd[64]; snprintf(cmd, sizeof(cmd), "AT+CMGS=\"%s\"", number); // Send the command, wait for '>' prompt char tmp[256]; snprintf(tmp, sizeof(tmp), "%s\r", cmd); write(fd, tmp, strlen(tmp)); // Read until '>' appears (modem ready for text) char buf[512] = {0}; long deadline = now_ms() + 5000; int total = 0; while (now_ms() < deadline) { int n = read(fd, buf + total, sizeof(buf) - total - 1); if (n > 0) { total += n; buf[total] = 0; } if (strchr(buf, '>')) break; usleep(20000); } if (!strchr(buf, '>')) return -1; // no prompt = error // Send the message body + Ctrl-Z write(fd, text, strlen(text)); write(fd, CTRL_Z, 1); // Wait for +CMGS: <ref>\r\nOK memset(buf, 0, sizeof(buf)); total = 0; deadline = now_ms() + 60000; // SMS can take up to 60 s while (now_ms() < deadline) { int n = read(fd, buf + total, sizeof(buf) - total - 1); if (n > 0) { total += n; buf[total] = 0; } if (strstr(buf, "+CMGS:") && strstr(buf, "OK")) return 0; if (strstr(buf, "ERROR")) return -1; usleep(20000); } return -1; // timeout }

⚠ WARNING: 60-second send timeout

SMS submission can take up to 60 seconds on congested networks. Do NOT use a short timeout or you will cancel mid-submit. Always wait for +CMGS: <ref> followed by OK before declaring success.

## 4.4 Receiving SMS — URC Direct Routing

With AT+CNMI=2,2,0,0,0 set, incoming messages are delivered directly to the serial port as +CMT URCs. You never have to read storage — the message appears inline in your read loop.

+CMT URC format (Text mode)

+CMT: "+447700900002","","26/01/15,12:30:45+00" Hello from the caller // Line 1: +CMT: "<sender>","<name>","<timestamp>" // Line 2: the message body

C — parse incoming +CMT URC

// In your URC read loop, when you see a line starting with +CMT: void handle_cmt(const char *cmt_line, const char *body_line) { // Parse sender from +CMT: "<number>","<alpha>","<time>" char sender[32] = {0}; const char *q1 = strchr(cmt_line, '"'); if (q1) { const char *q2 = strchr(q1 + 1, '"'); if (q2) { int len = q2 - q1 - 1; if (len < 32) { memcpy(sender, q1+1, len); sender[len] = 0; } } } // body_line is the next line read after the +CMT header ui_show_sms(sender, body_line); storage_save_sms(sender, body_line); }

## 4.5 Reading SMS from Storage — AT+CMGL

If you set AT+CNMI to store messages (storage mode) instead of routing them directly, use AT+CMGL to read them.

List and read stored SMS

AT+CMGL="ALL" ; list all messages (ALL, REC UNREAD, REC READ, STO UNSENT) AT+CMGR=1 ; read message at index 1 AT+CMGD=1 ; delete message at index 1 AT+CMGD=1,4 ; delete all messages (flag 4 = delete all)

+CMGL response format (Text mode)

+CMGL: 1,"REC READ","+447700900002","","26/01/15,12:30:45+00" Hello from the caller +CMGL: 2,"REC UNREAD","+447700900003","","26/01/15,13:00:00+00" Another message OK

## 4.6 Unicode SMS (Emoji and Non-Latin Scripts)

Send Unicode SMS

AT+CSCS="UCS2" ; switch to UCS2 hex encoding AT+CMGS="+447700900001" > 0048006500600006C006F00202764 ; 'Hello ❤' in UCS2 hex pairs <CTRL-Z> // Each character = 4 hex digits (big-endian UTF-16) // 'H' = 0048, 'e' = 0065, 'l' = 006C, 'o' = 006F // '❤' = 2764 // Helper: convert UTF-8 to UCS2 hex string in C // Use iconv(3): iconv_open("UCS-2BE", "UTF-8")

## 4.7 SMS Storage Capacity and SIM Memory

Check storage status

AT+CPMS? // Response: +CPMS: "SM",3,20,"SM",3,20,"SM",3,20 // used,total for each of: read, write, receive storage // If SIM storage is full, incoming SMS will be silently dropped! // Always check capacity and delete old messages. AT+CMGD=1,4 ; delete ALL messages to clear space

✖ CRITICAL: Full SIM memory drops incoming SMS

When SIM message storage is full, the network silently drops new incoming messages. Always monitor AT+CPMS? and delete read messages. In Black Hand OS, blackhand-storage should mirror SMS to /data/notes/ and purge the SIM storage.

# 5. Unsolicited Result Codes (URCs)

The modem sends URCs at any time — not as a response to a command. Your read loop must handle both synchronous AT responses and asynchronous URCs on the same serial port.

## 5.1 Key URCs to Handle

| URC | Trigger |
| --- | --- |
| RING | Incoming voice call (repeats every ~4 s) |
| +CLIP: <num>,<type>,... | Caller ID for incoming call |
| +CLCC: ... | Active call list (can arrive spontaneously) |
| NO CARRIER | Call dropped / remote hung up |
| BUSY | Remote line busy (outgoing call) |
| NO ANSWER | Outgoing call not answered |
| +CMT: ... | Incoming SMS (direct routing mode) |
| +CMTI: "SM",<idx> | Incoming SMS stored at <idx> |
| +CREG: <stat> | Network registration changed |
| +CGREG: <stat> | GPRS registration changed |
| +CSQ: <rssi>,<ber> | Signal quality report |

## 5.2 URC Reader Architecture

The challenge is that AT command responses and URCs arrive on the same port. The standard pattern is a dedicated reader thread that buffers lines and classifies them.

C — URC dispatcher pattern (simplified)

// Run this in a dedicated pthread void *modem_reader_thread(void *arg) { int fd = *(int *)arg; char line[512]; while (s_running) { if (!read_line(fd, line, sizeof(line), 500)) continue; // AT command responses go to the pending command waiter if (s_pending_command) { if (strstr(line, "OK") ¦¦ strstr(line, "ERROR") ¦¦ strstr(line, "+CME") ¦¦ strstr(line, "+CMS")) { cmd_response_append(line); cmd_response_signal(); // wake at_cmd() caller continue; } } // Classify URCs if (strcmp(line, "RING") == 0) on_ring(); else if (strncmp(line, "+CLIP:", 6) == 0) on_clip(line); else if (strcmp(line, "NO CARRIER") == 0) on_call_ended(); else if (strcmp(line, "BUSY") == 0) on_busy(); else if (strncmp(line, "+CMT:", 5) == 0) { // Next line is the SMS body char body[512]; read_line(fd, body, sizeof(body), 2000); on_sms_received(line, body); } } return NULL; }

# 6. Black Hand OS Integration

In Black Hand OS, modem functionality is split across two packages: blackhand-modem handles AT communication, call state, and SMS I/O; blackhand-ui presents it to the user via the notcurses TUI. They communicate over the IPC socket managed by blackhand-ipc.

## 6.1 Package Architecture

Package dependency graph

blackhand-ipc (shared IPC socket + message protocol) ├── blackhand-modem (AT serial driver, call/SMS engine) ├── blackhand-audio (ALSA mixer, call audio routing) └── blackhand-ui (notcurses TUI, receives events from modem)

## 6.2 blackhand-modem — Recommended Structure

src/ file layout for blackhand-modem

src/ ├── main.c — entry point, signal handling, service loop ├── serial.c/.h — open/close/read/write serial port, line buffering ├── at.c/.h — at_cmd(), async AT with timeout ├── modem_init.c/.h — startup sequence (ATE0, CMGF, CNMI, CLIP, etc.) ├── call.c/.h — dial, answer, hang_up, CLCC state machine ├── sms.c/.h — sms_send(), sms_receive(), sms_list() ├── urc.c/.h — URC dispatcher, reader thread └── ipc_bridge.c/.h — forward events to blackhand-ipc

## 6.3 Modem Initialization Sequence

Full modem init — run once after opening port

void modem_init(int fd) { // Basic setup at_cmd(fd, "ATE0", 2000); // echo off at_cmd(fd, "ATQ0", 2000); // result codes on at_cmd(fd, "ATV1", 2000); // verbose results at_cmd(fd, "AT+CMEE=2", 2000); // extended error text // SMS at_cmd(fd, "AT+CMGF=1", 2000); // text mode at_cmd(fd, "AT+CSCS=\"GSM\"", 2000); // GSM charset at_cmd(fd, "AT+CNMI=2,2,0,0,0", 2000); // direct URC delivery at_cmd(fd, "AT+CPMS=\"ME\",\"ME\",\"ME\"", 2000); // modem memory // Calls at_cmd(fd, "AT+CLIP=1", 2000); // caller ID enabled at_cmd(fd, "AT+CLVL=4", 2000); // speaker volume at_cmd(fd, "AT+CMUT=0", 2000); // mic unmuted // Network registration reporting at_cmd(fd, "AT+CREG=1", 2000); // enable +CREG URCs at_cmd(fd, "AT+CEREG=1", 2000); // enable +CEREG URCs }

## 6.4 IPC Message Protocol (call and SMS events)

blackhand-modem publishes events to blackhand-ipc. Define a simple message struct in the shared IPC header:

ipc_messages.h — modem events

typedef enum { MSG_CALL_INCOMING = 0x10, // payload: caller number string MSG_CALL_ACTIVE = 0x11, // payload: empty MSG_CALL_ENDED = 0x12, // payload: empty MSG_CALL_BUSY = 0x13, // payload: empty MSG_SMS_RECEIVED = 0x20, // payload: sender\x00body MSG_SMS_SENT_OK = 0x21, // payload: ref number MSG_SMS_SEND_FAIL = 0x22, // payload: error string MSG_SIGNAL_QUALITY = 0x30, // payload: rssi (0-31) MSG_NET_REG = 0x31, // payload: status (0-5) } ModemMsgType; // blackhand-ui subscribes to these and updates the TUI accordingly. // blackhand-ui sends commands back: typedef enum { CMD_DIAL = 0x50, // payload: number string CMD_ANSWER = 0x51, // payload: empty CMD_HANGUP = 0x52, // payload: empty CMD_SEND_SMS = 0x53, // payload: number\x00body CMD_DTMF = 0x54, // payload: digit char } ModemCmdType;

# 7. AT Command Quick Reference

## 7.1 Calls

| Command | Description |
| --- | --- |
| ATD<num>; | Dial voice call (semicolon required) |
| ATA | Answer incoming call |
| ATH | Hang up / reject |
| AT+CHUP | Hang up (explicit) |
| AT+CLCC | List current calls and states |
| AT+CHLD=0 | Release waiting/held calls |
| AT+CHLD=1 | Release active, accept held/waiting |
| AT+CHLD=2 | Hold active, answer waiting |
| AT+CHLD=3 | Conference: merge active + held |
| AT+VTS=<d> | Send DTMF digit during call |
| AT+CPAS | Activity status: 0=idle,3=ringing,4=in call |
| AT+CLIP=1 | Enable caller ID URCs |
| AT+CLVL=<0-5> | Speaker volume level |
| AT+CMUT=<0¦1> | Mic mute: 0=unmuted, 1=muted |
| AT+CSDVC=<1¦3> | Audio device: 1=handset, 3=handsfree |

## 7.2 SMS

| Command | Description |
| --- | --- |
| AT+CMGF=1 | Text mode (use this) |
| AT+CMGF=0 | PDU mode |
| AT+CSCS="GSM" | GSM 7-bit charset (standard) |
| AT+CSCS="UCS2" | Unicode charset (emoji, non-Latin) |
| AT+CMGS="<num>" | Send SMS to number, wait for > prompt |
| AT+CMGL="ALL" | List all stored messages |
| AT+CMGR=<idx> | Read message at index |
| AT+CMGD=<idx> | Delete message at index |
| AT+CMGD=1,4 | Delete all messages |
| AT+CPMS? | Check storage usage (used/total) |
| AT+CNMI=2,2,0,0,0 | Route new SMS directly to serial port |
| AT+CNMI=1,2,0,0,0 | Store new SMS, send +CMTI notification |

## 7.3 Network and Status

| Command | Description |
| --- | --- |
| AT+CSQ | Signal quality: +CSQ: <rssi>,<ber> (rssi 0-31, 99=no signal) |
| AT+CREG? | GSM registration: 0=idle,1=home,5=roaming |
| AT+CGREG? | GPRS registration |
| AT+CEREG? | LTE registration |
| AT+COPS? | Current operator name |
| AT+COPS=? | Scan all available operators |
| AT+CIMI | Read IMSI from SIM |
| AT+CGSN | Read modem IMEI |
| AT+CNUM | Read own phone number from SIM |
| AT+CFUN=1 | Full functionality mode |
| AT+CFUN=0 | Minimum functionality (radio off) |
| AT+CFUN=1,1 | Soft reset modem (re-reads SIM) |
| ATZ | Reset to factory defaults |
| AT&W | Save current settings |

# 8. Debugging and Common Issues

## 8.1 Debugging with minicom

Interactive AT session from Pi

# Install on buildroot by adding BR2_PACKAGE_MINICOM=y minicom -D /dev/ttyUSB0 -b 115200 --noinit --color=on # Or use busybox microcom: microcom -s 115200 /dev/ttyUSB0 # Or raw with stty + cat/echo: stty -F /dev/ttyUSB0 115200 raw cat /dev/ttyUSB0 & echo -e 'AT\r' > /dev/ttyUSB0

## 8.2 Common Errors Table

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| No response to AT | Wrong port or modem not powered | Check /dev/ttyUSB*, try all ports |
| +CME ERROR: 10 | SIM not found | Check SIM insertion, try AT+CFUN=1,1 |
| +CME ERROR: 11 | SIM PIN required | AT+CPIN=<PIN> |
| +CME ERROR: 30 | No network service | Check antenna, AT+COPS=? |
| +CMS ERROR: 330 | SMSC address not set | AT+CSCA="+<smsc_number>" |
| ATD returns ERROR | No network / modem not registered | Check AT+CREG?, wait for registration |
| SMS prompt > but no +CMGS | Message too long or encoding error | Reduce length, check AT+CSCS |
| RING but ATA gives ERROR | Call ended before answer | Answer faster / check timing |
| Audio silent on call | Audio routing not set | AT+CSDVC=1 or check PCM config |
| Modem port disappears | USB reset / kernel crash | Check dmesg, unplug/replug or reboot |

## 8.3 Signal Quality Interpretation

AT+CSQ response: +CSQ: <rssi>,<ber>

rssi 0 = -113 dBm or less (no signal) rssi 1 = -111 dBm rssi 2-30 = -109 to -53 dBm (normal range) rssi 31 = -51 dBm or greater (excellent) rssi 99 = unknown / no signal ber = bit error rate (0-7, 99=unknown) — usually 99 on LTE Practical thresholds: < 10 = poor — calls may drop 10-19 = fair — usable 20-31 = good to excellent

## 8.4 SIM PIN Handling

Check and unlock SIM PIN

AT+CPIN? ; check SIM status ; +CPIN: READY — no PIN needed ; +CPIN: SIM PIN — PIN required ; +CPIN: SIM PUK — PUK required (too many wrong PINs) AT+CPIN=1234 ; enter PIN AT+CLCK="SC",0,"1234" ; disable PIN lock permanently

## 8.5 Checking Port Detection on Pi

Find the right /dev/ttyUSB port

# List all ttyUSB ports ls -la /dev/ttyUSB* # Check which port is the SIM7600 AT port for port in /dev/ttyUSB0 /dev/ttyUSB1 /dev/ttyUSB2 /dev/ttyUSB3; do echo -n "Testing $port: " stty -F $port 115200 raw echo -e 'AT\r' > $port timeout 1 cat $port ¦ tr -d '\r' ¦ head -1 done # In dmesg, look for: # [ 5.123] usb X-Y: GSM modem (1-port) converter now attached to ttyUSB0

✓ TIP: Persistent device naming

To prevent port numbers from shifting when other USB devices are plugged in, create a udev rule: SUBSYSTEM=="tty", ATTRS{idVendor}=="1e0e", ATTRS{idProduct}=="9001", SYMLINK+="ttyMODEM". Then open /dev/ttyMODEM in blackhand-modem. SIMCom vendor ID is 1e0e; confirm with lsusb.

# 9. Implementation Checklist

## 9.1 Phase A — Serial Communication

- Open /dev/ttyUSB0 at 115200 baud, 8N1, raw mode

- Send ATE0, wait for OK — verify modem alive

- Implement at_cmd() with configurable timeout

- Implement line reader with \r\n stripping

- Start URC reader thread

## 9.2 Phase B — Initialization

- Run modem_init() sequence (echo off, text SMS, CLIP, CNMI)

- Read IMEI, IMSI, own number — store in modem state

- Check network registration (+CREG?)

- Handle SIM PIN if needed (+CPIN?)

## 9.3 Phase C — Voice Calls

- Implement dial(fd, number) — ATD;

- Implement answer(fd) — ATA

- Implement hang_up(fd) — ATH

- Handle RING, NO CARRIER, BUSY URCs

- Implement AT+CLCC polling to track call state

- Publish call events to blackhand-ipc

## 9.4 Phase D — SMS

- Implement sms_send(fd, number, text) with > prompt detection

- Handle +CMT URC for incoming SMS (direct routing)

- Parse sender number from +CMT header

- Save received SMS to /data/notes/ via blackhand-storage

- Monitor AT+CPMS? and clear storage when near full

## 9.5 Phase E — UI Integration

- blackhand-ui subscribes to MSG_CALL_INCOMING → show incoming call screen

- Dial / answer / hangup buttons send CMD_DIAL, CMD_ANSWER, CMD_HANGUP to blackhand-modem

- SMS compose screen sends CMD_SEND_SMS

- Signal strength bar polls MSG_SIGNAL_QUALITY every 10 s

- Network status indicator shows MSG_NET_REG

This completes the core implementation. Extended features to add in Phase 3: USSD codes (AT+CUSD), call forwarding (AT+CCFC), multi-party conferencing (AT+CHLD=3), and LTE data (PPP / MBIM via QMI).